POLICY TITLE: EQUIPMENT, SUPPLIES AND DIALY DRUG CHECKS

POLICY:

To ensure quality care to all patients undergoing anesthesia, all equipment, supplies, and drugs will be checked daily by the Anesthesia staff.

PROCEDURE:

- Check all anesthesia machines:

- O2 tanks on machines are to be checked daily.  If below 5000, replace with a new tank.

- Replace all nitrous oxide tanks below 500.

- Rubber tubing and hoses for cracks and wearing.

- Evacuation tubing.

- Oxygen analyzers (batteries, fluid, and membrane) are checked each day.  If batteries are low, replace with three (3) batteries.  Check membrane and PO2 fluid each week and replace if needed.

- Sodasorb should be changed when blue in color.

- Clean canisters with soap and water.  Let drain well, then refill with fresh Sodasorb.  Label and date new canister.

- Canisters containing Sevoflorane are to be checked once each month for any discoloration of fluid.  If yellow, drain canister and refill with 100 cc Sevoflorane.

- Stock carts in each O.R. suite before and at end of shift.

- Stock anesthesia machines in each O.R. suite before and at end of shift.

- Set up each O.R. suite for early morning.

- Replace stock drugs in proper areas.