### POLICY TITLE: MONITORING OXYGEN SATURATION

PURPOSE:

The purpose of this policy is to establish guidelines that ensure that all sedated patients have their oxygen saturation monitored by use of a pulse oximeter and to establish criteria for interpreting the reading.

POLICY:

All sedated patients will receive oxygen via nasal cannula and their oxygen saturation monitored by use of a pulse oximeter intra and post procedure.

PROCEDURE:

- Turn oximeter to "ON" and verify alarm limit for saturation and pulse.

- Apply transducer according to manufacturer's directions to one of the following:

  - Index finger

  - Toe

- Observe oxygen saturation and pulse on digital display of oximeter.

- 02 saturation parameters:

  - 94-100% is within normal limits.

  - Below 94% notify physician and increase liter flow if ordered.

  - Less than 90% needs ventilatory intervention if verbal or tactile stimulation does not raise the oxygen saturation to 90% or above.

- Remove transducer post procedure when saturation is within the normal limits.

- Documentation:

  - Patient Care Notes:

    - Document oxygen saturation values on admission to the facility, every five minutes, 	during procedure and at termination of procedure. A monitor strip, oxygen saturation 	display, may be attached to the chart.

    - Post Procedure, oxygen saturation values are recorded every five minutes times three or 	until the patient is stable.

- Guidelines/Precautions:

  - Transducer must be appropriate to patient's size. If not, accurate readings will not be obtained.

  - Patient movement will interfere with sensor performance. Hold sensor still to obtain accurate values. Do not place the sensor on the same extremity as the blood pressure cuff.

  - Protect sensor from light. Strong light source can interfere with values and give an artificially low reading.