### POLICY TITLE: MAINTENANCE OVERVIEW

PURPOSE:

The purpose of this policy is to establish guidelines for the proper and timely maintenance of the building’s mechanical equipment.

POLICY:

The building and healthcare equipment is properly inspected and maintained according to the manufacturer’s instructions.

PROCEDURE:

- The biomedical company will inspect all equipment used directly on patients on a twice annually, or at a frequency determined by the equipment’s instructions for use.  A report is issued to the Director of Nursing. The equipment inventory is updated with each inspection.

- The following facility mechanical systems are monitored, serviced and maintained. Contracts are in place to define timing of maintenance and service.

  - HVAC System: the air supply meets minimum requirements for filtration, humidity and temperature control.

    - The HVAC filters are changed at least annually. More frequent changes may be necessary based on recommendations of service personnel. The system is maintained twice per year.

  - Lighting: Glare free lighting is provided. Bulbs are changed as needed.

  - Fire alarm system is serviced and tested quarterly.

  - The nurse call system is tested quarterly.

  - The medical gas manifold and lines are tested and maintained twice per year. Regular checks of the medical gas levels are conducted by the Director of Nursing or designee.

  - The emergency generator is serviced annually.

    - The generator is checked weekly to ensure that the generator has performed a self-check 	before or after business hours. Battery will be inspected for corrosion.

    - Monthly, the generator is run at full load by manually turning off the main electricity 	supply switch to the facility and observing for the immediate generator start-up. This may be an automated function.  Annually the generator is then run for 90 minutes or per guidelines for individual unit.

    - The patient monitors are equipped with backup battery power.

    - Battery packs with large bulbs are positioned throughout the rooms and corridors of the 	facility.

    - The EXIT signs are equipped with backup battery power.

    - There are battery backup lights inside the ceiling lighting in the operating rooms.

    - Flashlights are available throughout the facility.

  - All emergency lighting is checked monthly and logged.

  - All maintenance forms, completed by the contracted maintenance companies are kept on file in the Director of Nursing office.

  - All routine checks of equipment by staff are logged and maintained.

  - Any deviation or problems with any equipment check is reported immediately to the Director of Nursing who will contact the company responsible for the equipment maintenance.