POLICY:  BORROWED INSTRUMENTS AND EQUIPMENT

PURPOSE: To outline procedures for the management of loaner instruments/ equipment to ensure sterility, proper function, and safe use of the instruments/ equipment.

PROCEDURE: Loaner instruments/ equipment will be inventoried, examined, cleaned, and sterilized by Surgery Center XYZ before use, following all manufacturers’ written instructions.  All loaner electrical equipment will be checked and approved by a bio-medical engineer prior to use.

- At the time the procedure is scheduled, XYZ will:

- Request or order the instruments so there will be sufficient time for processing.

- Before receiving loaner instruments, XYZ will:

- Obtain and evaluate the manufacturer’s instructions for use for reprocessing and handling to determine the ability of XYZ to clean and reprocess the instruments adequately.

- Obtain any accessories necessary for reprocessing.

- Request an inventory list be sent with the tray to verify the set is complete.

- Prior to use of loaner instruments / equipment at XYZ, XYZ will:

- Log in and inventory the instruments/ equipment.

- Verify the set of instruments/ equipment is complete.

- Examine the instruments/ equipment, including moving parts, tips, box locks, ratchets, and screws for defects.

- Decontaminate and sterilize the instruments/ equipment according to all manufacturers’ instructions for use and facility policies.

- Following the procedure at XYZ, XYZ will:

- Disassemble and decontaminate the instruments/ equipment.

- Inventory the instruments/ equipment to ensure completeness of the set

- Document disposition of the instruments/ equipment.