POLICY: HANDLING OF STERILE SUPPLIES

PURPOSE: The purpose of the sterile storage area, regardless of the location, is to safely store                    sterile items and prevent contamination until use.

PROCEDURE: The following guidelines will be followed for the handling and storage of sterile supplies:

- Outside shipping containers and corrugated cartons will not be used as containers in sterile storage or other storage areas in patient-care areas of the facility.

- Sterile supplies will be unloaded in the receiving area and placed on a cart for transport or on the shelf.

- Supplies will be examined for any compromise of the package integrity (tears, punctures, moisture, etc.) prior to placing them on the cart for transport to the restricted area.

- Damaged supplies will remain in the receiving area until a manager is notified and exchanges can be arranged.

- Supplies will be transported to their storage area via the cart.

- Sterile supplies will be primarily located in the closed cabinets, containers, and drawers of the supply room and in the operating rooms, but may also be stored in various designated areas in the pre- and post-operative areas.  If sterile supplies are not stored in an enclosed cabinet, they must be covered with a water-proof barrier because there is potential for contamination when in non-controlled sterile storage areas.

- Sterile supplies will be grouped together in clearly marked drawers/cabinets throughout the center.  Sterile supplies will be stored above non-sterile supplies.

- Items will be stored in a manner that does not compromise sterility, i.e. next to or under sinks or in any location where there is a likelihood or potential for high levels of moisture.

- Paper products will be stored above liquid products.

- The shelf life of sterilized items will have two designated standards:

- Pre-packaged sterile items will be considered sterile unless there is an event-related incident that renders them unsterile, i.e., a puncture or tear in the packaging, moisture in the packaging, etc.

- Upon reaching manufacturer’s expiration date.

- At this time, no single-use items will be sterilized in the center. Disposable, single-use instruments may be utilized.  There will be no re-sterilizing and repackaging of single-use devices.

- The area where sterile supplies are stored must be monitored for temperature and humidity on a daily basis.  The temperature must be between 72° F and 78° F and the humidity a maximum of 60%.

- Supplies cannot be stored within 8 inches of the floor, 18 inches from the ceiling, and 2 inches from external walls.

- The bottom shelf on portable shelving units must be covered or solid.

- Portable shelving units need to be covered with a water-proof cover at all times.

Reference:

  - CDC: Guideline for Disinfection and Sterilization in Healthcare Facilities, 2008